变物性与收缩条件下药材烘干的传热传质模型

摘  要

药材烘干时，热量从表面向内部传递，水分从内部向外迁移。两者的变化速度并不相同。物性变化和几何收缩还会改变干燥终点。我们据此建立圆柱形药材的一维径向传热传质模型，用环形有限体积法和隐式时间积分求解，并用 Richardson 外推核对数值精度。

关于问题一，在固定半径和常热物性条件下，分别求解温度场和含水率场。计算得到 1800 s 时中心、表面温度分别为 33.5753 ℃和 36.7856 ℃，对应含水率分别为 2.5500 kg/kg 和 1.5102 kg/kg。由此可见，预热阶段中心水分变化很小，失水主要集中在表层。

关于问题二，将题设中的物性参数表示为温度和含水率的函数，建立温湿耦合模型。3 h 时中心、表面温度分别为 49.8495 ℃和 49.9664 ℃，含水率分别为 1.7662 kg/kg 和 1.0081 kg/kg。可以看出，此时温度已经接近均匀，但水分仍有明显的径向梯度，升温完成后还需要继续干燥。

关于问题三，以附件 1 最后一小时的环境均值延拓长期边界，并把全域最大含水率低于 0.15 kg/kg 作为干燥完成的判据。固定半径模型的临界时长为 57.4740 h，首个严格达标整分钟为 57.4833 h。在不同长期环境设定下，临界时长约为 57.24—57.83 h，说明结果对环境延拓方式较为敏感。

关于问题四，根据均匀收缩条件下的干物质和水分守恒建立移动边界模型，并用归一化径向坐标把变化区域映射到固定区间。计算得到临界时长为 51.0920 h，首个严格达标整分钟为 51.1000 h。若保持本问物性而固定初始半径，对照时长为 129.8481 h，说明收缩会明显加快本模型中的干燥过程。

最后，对网格、通量平衡和独立程序进行检验，结果均支持上述计算。两类终点的相邻外推值相差约 1 s。所得时长是在一维近似、有效传质边界和经验密度—收缩关系等条件下得到的预测值。

关键词：药材烘干；传热传质；有限体积法；移动边界；干燥终点

# 1 问题重述与分析

## 1.1 任务及数据

题目研究一根长 25 cm、初始半径为 2 cm 的圆柱形药材。烘干开始时，药材温度为 28 ℃，干基含水率为 2.55 kg/kg。附件 1 给出了烘房初期的温度和含湿量，附件 2 给出了药材半径的变化。题目既要求整体干燥情况，也要求指定位置的温度和含水率。我们不能只计算一个平均值，必须描述药材内部的变化 [1]。

四个问题分别考察预热过程、变物性干燥、干燥终点和几何收缩。问题一计算固定半径下 30 min 内的温度和含水率；问题二从题设初值出发，采用附录 3 物性计算全程温湿分布，并列出前三小时结果；问题三沿用问题二模型，确定各处含水率均低于 0.15 kg/kg 的时间；问题四采用附录 4 物性和实测半径，重新计算干燥终点。问题二从初始时刻独立计算，问题三是在此基础上的长期延伸。

## 1.2 模型选择

考虑到题目既要求内部局部分布，又要求全域达标时间，只用平均含水率曲线不能满足要求。我们用 Fourier 导热方程描述传热，用 Fick 有效扩散方程描述水分迁移，再用表面对流边界连接药材和烘房环境。圆柱物料的热质耦合模型 [2] 和非等温移动边界模型 [3] 为建模提供了参考。物性和观测数据均采用题目给定值。

药材长径比为 6.25。我们先忽略中段的轴向差异，只计算半径方向的变化，并将结果看作圆柱中截面的分布。建模时，先处理常物性情形，再考虑物性变化，最后加入实测收缩。问题四直接使用附件给出的半径数据，不再另外拟合收缩方程。

# 2 模型假设与符号

## 2.1 基本假设

（1）药材为径向各向同性的轴对称连续介质，忽略周向差异和中段轴向梯度，以一维径向场近似中截面分布。

（2）第一至第三问半径不变；第四问长度不变、截面均匀径向收缩，整体半径历史适用于所研究的中截面。

（3）初始温度和干基含水率均匀。热物性与扩散率按各问指定附录取局部状态值，不另行拟合参数。

（4）附件 1、附件 2 均作分段线性插值。4 h 后的烘房环境以最后一小时均值为基准平台，末观测之后用 60 s 过渡到平台；这是长期环境假设。

（5）第二至第四问未另给表面系数，故沿用第一问的传热系数 25 W/(m²·K) 和传质系数 8×10⁻⁷ m/s。气相含湿量与固相干基含水率的质量基准不同，在缺少吸附等温线时，将二者之差解释为有效传质推动力。

（6）忽略蒸发潜热、热湿交叉迁移和力学应力，以经验密度和比热的乘积表示有效体积热容。模型按有效介质解释，其质量与热力学闭合限制见第 8.5 节。

（7）第四问干物质无损失、初始干物质体积密度均匀；在均匀收缩下，该干密度只随时间变化。此处的守恒干密度与附录经验密度分开定义，二者的相容性在第 8 节检验。

## 2.2 主要符号

为便于阅读，我们先列出模型的主要状态变量、物性参数和终点时刻。这里的下标 ∞ 表示烘房环境，下标 s 表示药材实际表面。

表 2.1 主要状态变量与物性参数说明

| 符号 | 含义 | 单位 |
| --- | --- | --- |
| t | 自烘干起点计的时间 | s |
| r,R(t),R0 | 物理半径、当前表面半径、初始表面半径 | m |
| ξ=r/R(t) | 随材料运动的归一化径向坐标 | 1 |
| T,TK | 摄氏温度、开尔文温度 | ℃、K |
| C | 固相干基含水率 | kg/kg |
| T∞,C∞ | 烘房温度、环境含湿量 | ℃、kg/kg |
| ρ,cp,k | 经验密度、比热容、导热系数 | kg/m³、J/(kg·K)、W/(m·K) |
| D | 有效水分扩散率 | m²/s |
| hT,hm | 对流传热、有效传质系数 | W/(m²·K)、m/s |
| ρd,vr | 当前干物质体积密度、径向骨架速度 | kg/m³、m/s |
| t*,t60 | 临界时刻、其后首个严格达标整分钟 | s |

## 2.3 离散变量与中间量

在有限体积计算中，同一符号还会带有单元下标或界面下标。为避免将几何量、通量和终点判据混淆，补充说明如表 2.2。

表 2.2 离散计算与诊断中的符号说明

| 符号 | 含义 | 单位 |
| --- | --- | --- |
| i,N | 径向控制体编号、径向控制体总数 | 1 |
| ri−1/2,ri+1/2 | 第 i 个控制体内、外界面的物理半径 | m |
| Δr | 固定半径模型的径向网格间距 | m |
| Vi,Ai+1/2 | 单位轴向长度下第 i 个控制体体积、外界面面积 | m²、m |
| u,s,Γ | 一般守恒式中的传输变量、蓄积系数、输运系数 | 随所取方程而定 |
| f,fi,ti | 插值变量、相邻观测点的变量值及其时刻 | 随变量而定、s |
| z | 为保证正性而引入的对数含水率，z=lnC | 1 |
| uext | 由两级网格结果作 Richardson 外推后的量 | 与 u 相同 |
| C | 截面面积加权平均干基含水率 | kg/kg |
| g(t) | 全域最大含水率与 0.15 kg/kg 阈值的差 | kg/kg |
| v,jw | 骨架速度、水分相对骨架的扩散通量 | m/s、kg/(m²·s) |
| ρd0,Md,L | 初始干密度、总干物质量和药材长度 | kg/m³、kg、m |
| R˙ | 表面半径对时间的导数 | m/s |

所有计算距离使用 m，表格按题意显示 cm；内部温度保存为 ℃，只在扩散率的 Arrhenius 项中换算为 K。后文的“水分浓度”均指题目定义的干基含水率。至此，后文公式和表格中出现的主要变量均已给出定义。

# 3 数据准备与统一数值方法

## 3.1 数据核验及环境延拓

附件 1 共有 241 组记录，时间范围为 0—14400 s，采样间隔为 60 s；附件 2 共有 145 组记录，时间范围为 0—259200 s，采样间隔为 1800 s。经核对，提取后的两份 CSV 与原始工作簿逐值相同。药材半径由 2.000 cm 逐渐减小到 1.198 cm，计算时直接使用原始观测值，不另用拟合曲线代替。

观测点之间没有直接给出数值。我们先按相邻两点的连线补足中间时刻。设 fi,fi+1 分别为相邻时刻的环境量或半径，比例插值得

f(t)=fi+t−titi+1−ti(fi+1−fi),ti≤t≤ti+1	(1)

附件 1 只记录了前 4 h 的烘房环境，而问题三、四需要更长时间的环境条件。我们取最后一小时 61 个观测点的均值作为后续环境平台。该时段温度和含湿量的均值分别为 49.998934 ℃和 0.04998754 kg/kg，标准差分别为 0.154353 ℃和 0.00015013 kg/kg；相应线性趋势约为 0.002824 ℃/h 和 −0.00001288 kg/(kg·h)。可以看到，该时段波动较小。至此，模型中的环境参数取值已知，可以用于后续计算。第 8.3 节再比较不同延拓方法对结果的影响。

## 3.2 环形有限体积离散

为计算温度和含水率在半径方向上的变化，我们把药材截面划分为若干同心圆环。省略各单元共有的轴向长度后，第 i 个控制体的体积为 Vi=π(ri+1/22−ri−1/22)，外侧面积为 Ai+1/2=2πri+1/2。对每个圆环，先写“单元内累积量 = 外侧流入量 − 内侧流出量”。设 u 为传输变量，s 为蓄积系数，Γ 为输运系数，就得到

siViduidt=Ai+1/2Γi+1/2ui+1−uiΔr−Ai−1/2Γi−1/2ui−ui−1Δr	(2)

式（2）左侧是圆环内变量的变化，右侧是两个界面的净通量。温度方程取 u=T,s=ρcp,Γ=k；水分方程取 u=C,s=1,Γ=D。相邻单元的物性不同，我们用调和平均计算界面系数：

Γi+1/2=2ΓiΓi+1Γi+Γi+1	(3)

圆心控制体的内侧面积为零，因此可满足 r=0 处的对称条件；最外层控制体采用题设的 Robin 边界。相邻控制体之间的通量大小相等、方向相反，所有单元求和后只保留表面通量，后文据此检验水分平衡。

## 3.3 时间积分、正性与外推

空间离散后，偏微分方程变为常微分方程组。我们用后向差分公式（BDF）推进时间。为避免出现负含水率，先令 z=lnC，按 zt=Ct/C 计算，再由 C=expz 还原含水率。积分器自动调整步长，并在题目要求的时刻输出结果。

为减小空间离散误差，我们在相同物理输出节点上计算 N 和 2N 两套网格。若误差主项近似与网格步长的平方成正比，两式相消后可得到二阶 Richardson 外推：

uext=4u2N−uN3	(4)

问题一采用 5120/10240 两级网格，问题二、三采用 2560/5120 两级网格，问题四采用 640/1280 两级网格。计算结果先按式（4）外推，再按题意保留四位小数。问题二每 600 s 分段计算，段与段之间传递未舍入的节点值。

# 4 问题一：固定半径预热模型

## 4.1 方程、初值与边界

问题一先不考虑收缩，计算区域固定为 0<r<R0=0.02 m。热量守恒给出导热方程；水分守恒并取有效扩散通量后给出扩散方程。因此我们采用

ρcp∂T∂t=1r∂∂rrk∂T∂r∂C∂t=1r∂∂rrD(C)∂C∂r	(5)

附录 2 直接给出 ρ=820 kg/m³、cp=2600 J/(kg·K)、k=0.36 W/(m·K)。水分扩散率随含水率变化，按题设关系取

D(C)=7×10−9exp0.89Cm2s	(6)

烘干刚开始时，内部温度和含水率均匀，故取 T(r,0)=28 ℃、C(r,0)=2.55 kg/kg。圆心两侧完全对称，故 Tr(0,t)=Cr(0,t)=0。在表面，传导或扩散通量应等于与空气的对流交换通量。径向向外取正，于是边界条件为

−kTr(R0,t)=hT[Ts−T∞(t)]−DCr(R0,t)=hm[Cs−C∞(t)]	(7)

由式（7）可知，当烘房温度高于药材表面温度时，热量由环境进入药材；当表面含水率高于环境有效含湿量时，水分由药材向外迁移。问题一中的热物性为常数，扩散率也不随温度变化，因此温度方程和水分方程可以分别求解。

## 4.2 指定位置的计算结果

将附件 1 的环境数据逐时刻代入上述模型，我们得到表 1、表 2。表中列出题目指定时刻和位置的结果。完整的 1—1800 s 温度场和含水率场保存在支撑材料 result1.xlsx 中，径向间隔为 0.1 cm，表中结果按题意保留四位小数。

表 1  代表位置温度 T/∘C

| 时间/s | 0 cm | 0.5 cm | 1.0 cm | 1.5 cm | 2.0 cm |
| --- | --- | --- | --- | --- | --- |
| 100 | 28.0001 | 28.0003 | 28.0040 | 28.0327 | 28.1801 |
| 300 | 28.0408 | 28.0635 | 28.1514 | 28.3681 | 28.8489 |
| 600 | 28.4533 | 28.5360 | 28.8039 | 29.3159 | 30.1652 |
| 900 | 29.3243 | 29.4583 | 29.8755 | 30.6161 | 31.7304 |
| 1200 | 30.5427 | 30.7098 | 31.2223 | 32.1126 | 33.4276 |
| 1500 | 31.9957 | 32.1867 | 32.7660 | 33.7463 | 35.1203 |
| 1800 | 33.5753 | 33.7720 | 34.3642 | 35.3621 | 36.7856 |

表 2  代表位置水分浓度 C/(kg/kg)

| 时间/s | 0 cm | 0.5 cm | 1.0 cm | 1.5 cm | 2.0 cm |
| --- | --- | --- | --- | --- | --- |
| 100 | 2.5500 | 2.5500 | 2.5500 | 2.5500 | 2.2470 |
| 300 | 2.5500 | 2.5500 | 2.5500 | 2.5492 | 2.0508 |
| 600 | 2.5500 | 2.5500 | 2.5500 | 2.5353 | 1.8770 |
| 900 | 2.5500 | 2.5500 | 2.5497 | 2.5045 | 1.7547 |
| 1200 | 2.5500 | 2.5500 | 2.5482 | 2.4646 | 1.6586 |
| 1500 | 2.5500 | 2.5499 | 2.5445 | 2.4206 | 1.5787 |
| 1800 | 2.5500 | 2.5497 | 2.5383 | 2.3755 | 1.5102 |

图 1 预热阶段的径向温度与含水率分布；不同曲线对应图例给出的时刻。

由图 1 可以看到，空间分布上温度从表面向中心逐渐降低；相同时刻，含水率从中心向表面逐渐降低。1800 s 时，表面与中心的温差为 3.2103 ℃；表面含水率已经降至 1.5102 kg/kg，中心含水率仍接近初值。我们据此判断，预热阶段温度传播快于内部水分迁移，药材表层先出现明显失水。

# 5 问题二：变物性温湿耦合模型

## 5.1 经验物性与耦合关系

问题二从烘干初始时刻重新计算。我们不再把物性看成常数，而是直接采用附录 3 给出的含水率关系 [1]：

ρ=650+128C,cp=1450+2736CC+1,k=0.21+0.38CC+1	(8)

D(C,T)=2.4×10−3exp0.45Cexp3850TK,TK=T+273.15	(9)

式（8）给出热方程所需的密度、比热和导热系数；式（9）给出水分方程所需的扩散率。把它们逐个代入式（5），再保留原初值和式（7）的边界条件，就得到问题二模型。含水率变化会改变密度、比热和导热系数。温度又会通过扩散率影响水分迁移。因此我们同时求解温度和含水率。计算 Arrhenius 项时，将摄氏温度换算为开尔文温度。至此，模型中的物性参数取值已知，可以用于计算温湿分布。

这里将水分迁移看作固定骨架中的有效扩散，并用经验量 ρ(C)cp(C) 表示体积热容。第 8.5 节会说明，若把 ρ(C)/(1+C) 直接作为守恒干物质密度使用，则会与收缩关系不一致。

## 5.2 前三小时结果与全程输出

表 3　3 小时内药材温度（°C）

| 时间/h | 0 cm | 0.5 cm | 1.0 cm | 1.5 cm | 2.0 cm |
| --- | --- | --- | --- | --- | --- |
| 0.5 | 32.1892 | 32.3821 | 32.9659 | 33.9608 | 35.4130 |
| 1.0 | 40.3816 | 40.5536 | 41.0605 | 41.8782 | 42.9976 |
| 1.5 | 45.8468 | 45.9348 | 46.1930 | 46.6051 | 47.1400 |
| 2.0 | 48.4502 | 48.4881 | 48.5984 | 48.7736 | 49.0033 |
| 2.5 | 49.4670 | 49.4792 | 49.5136 | 49.5653 | 49.6609 |
| 3.0 | 49.8495 | 49.8553 | 49.8746 | 49.9101 | 49.9664 |

表 4　3 小时内药材水分浓度（kg/kg）

| 时间/h | 0 cm | 0.5 cm | 1.0 cm | 1.5 cm | 2.0 cm |
| --- | --- | --- | --- | --- | --- |
| 0.5 | 2.5499 | 2.5489 | 2.5256 | 2.3257 | 1.6486 |
| 1.0 | 2.5257 | 2.4948 | 2.3578 | 2.0230 | 1.4711 |
| 1.5 | 2.3861 | 2.3256 | 2.1344 | 1.8020 | 1.3476 |
| 2.0 | 2.1709 | 2.1084 | 1.9236 | 1.6259 | 1.2311 |
| 2.5 | 1.9566 | 1.9006 | 1.7360 | 1.4720 | 1.1166 |
| 3.0 | 1.7662 | 1.7165 | 1.5702 | 1.3333 | 1.0081 |

图 2 变物性模型前三小时的径向温湿分布。

从表 3、表 4 和图 2 可以看到，空间分布上温度曲线已接近平坦，而含水率曲线仍由中心向表面明显下降。3 h 时中心与表面的温差只有 0.1169 ℃；含水率仍相差 0.7581 kg/kg，且各处都没有达到 0.15 kg/kg。随着表层含水率降低，扩散率继续减小，内部水分向外迁移还需要较长时间。我们因此不能把表面温度稳定作为烘干完成的依据。

表 3、表 4 给出了前三小时的计算结果。全程温湿场按 1 s 时间间隔、0.1 cm 径向间隔输出，并一直计算到问题三的首个严格达标整分钟。长期环境按第 3.1 节处理，具体输出和跨问题核对方式见附录 A.2。

# 6 问题三：固定半径的干燥终点

## 6.1 全域阈值与严格不等式

问题三要求药材各处的含水率都低于 0.15 kg/kg。我们先在每一时刻找出全部径向节点中的最大值，再用该最大值和阈值的差定义判别函数：

g(t)=max0≤r≤R0C(r,t)−0.15,t*=inf{t:g(t)<0}	(10)

式（10）中，g(t) 表示最湿位置与阈值的差；t* 是这个差首次变为负数的时刻。这样做保证了我们没有遗漏较湿的内部位置。计算结果表明，中心位置的含水率最高，所以干燥终点由中心位置决定。在临界时刻有 g(t*)=0，其后的时刻才严格低于阈值。

为确定终点，我们在阈值两侧对各径向位置的未舍入含水率作时间插值，取最晚达到阈值的位置。若设备只能按整分钟停止，就要把临界时刻向后推到下一个整分钟。因此

t60=60t*601	(11)

## 6.2 时长与指定结果

第三问临界时刻为 206906.2447 s（57.4740 h），首个严格达标整分钟为 206940 s（57.4833 h）。

表 5　药材烘干过程的水分浓度（kg/kg）

| 时间/h | 0 cm | 0.5 cm | 1.0 cm | 1.5 cm | 2.0 cm |
| --- | --- | --- | --- | --- | --- |
| 6 | 1.0170 | 0.9881 | 0.9015 | 0.7550 | 0.5328 |
| 12 | 0.4561 | 0.4432 | 0.4031 | 0.3297 | 0.1642 |
| 18 | 0.2988 | 0.2916 | 0.2687 | 0.2253 | 0.0845 |
| 24 | 0.2378 | 0.2327 | 0.2167 | 0.1855 | 0.0665 |
| 30 | 0.2058 | 0.2018 | 0.1892 | 0.1641 | 0.0598 |
| 36 | 0.1859 | 0.1825 | 0.1718 | 0.1504 | 0.0566 |
| 42 | 0.1721 | 0.1691 | 0.1597 | 0.1407 | 0.0548 |
| 48 | 0.1618 | 0.1592 | 0.1507 | 0.1334 | 0.0537 |
| 54 | 0.1539 | 0.1514 | 0.1436 | 0.1277 | 0.0530 |
| 临界终点 57.4740 | 0.1500 | 0.1477 | 0.1402 | 0.1249 | 0.0526 |

表 5 最后一行给出了临界时刻的含水率。此时中心显示为 0.1500，恰好位于阈值边界。按式（11）计算，下一整分钟才严格满足题目要求。完整工作簿同时保留了分钟序列、临界行和首个严格达标整分钟行。

图 3 固定半径下中心、中半径和表面的长期含水率历程；虚线为 0.15 kg/kg 阈值。

由图 3 可以看到，三条曲线从表面到中心依次上移，表面含水率下降最快，中心下降最慢。进入干燥后期后，各位置的变化都逐渐变缓。因此，只判断表面或平均含水率会提前结束烘干。我们必须以全域最大值作为终点判据。上述时长是在第 3.1 节长期环境假设下得到的，其敏感性将在第 8.3 节分析。

# 7 问题四：收缩移动边界模型

## 7.1 从干物质与水分守恒出发

问题四中药材半径随时间变化。附件 2 已经给出 R(t)，我们直接把实测半径代入模型。在均匀径向收缩、长度不变的条件下，离中心越远的材料移动越快，故材料速度为 vr=(R˙/R)r。设当前干物质体积密度为 ρd，相对骨架的水分通量为 jw。先对干物质和水分分别写守恒关系，得到

∂tρd+∇·(ρdv)=0∂t(ρdC)+∇·(ρdCv+jw)=0	(12)

第一式表示干物质没有生成或损失；第二式表示水分随骨架运动，还会相对骨架扩散。我们取有效 Fick 通量 jw=−ρdD∇C，再用第一式消去第二式中的干物质连续项，得到

DCDt=1ρd∇·(ρdD∇C)	(13)

初始干密度均匀，截面又作均匀径向收缩。单位长度的截面积正比于 R(t)2，所以干物质守恒给出 ρd(t)R(t)2=ρd0R02。此时干密度只随时间变化，式（13）右侧的 ρd 可以约去。将材料导数展开为 DC/Dt=∂C/∂t+vr∂C/∂r，便得到

∂C∂t+R˙Rr∂C∂r=1r∂∂rrD∂C∂r	(14)

式（14）中的 C 是单位干物质量所含的水分，几何收缩已经通过骨架运动和干物质守恒体现，因此不需要再加入体积压缩源项。

## 7.2 固定域坐标与表面通量

实际区域的边界不断移动，直接离散会反复改变网格。我们令 ξ=r/R(t)，把收缩区域变换到固定区间 0<ξ<1。固定 ξ 的网格速度与骨架速度相同，因此坐标变换项与式（14）中的对流项相互抵消。再按 ∂/∂r=R−1∂/∂ξ 变换两个径向导数，就得到

ρ(C)cp(C)∂T∂t|ξ=1R(t)2ξ∂∂ξξk(C)∂T∂ξ	(15)

∂C∂t|ξ=1R(t)2ξ∂∂ξξD(C,T)∂C∂ξ	(16)

式（15）来自热传导方程的同一坐标变换，式（16）来自式（14）的坐标变换。热方程仍采用有效热容近似，不计变形功和相变热。圆心满足对称条件 Tξ=Cξ=0。表面仍满足“内部通量 = 与空气的对流通量”，而 r 导数换成 ξ 导数后多出 1R，所以实际表面的边界条件为

−kRTξ(1,t)=hT(Ts−T∞)−DRCξ(1,t)=hm(Cs−C∞)	(17)

由式（15）—（17）可知，内部扩散项随 R−2 变化，表面通量随 R−1 变化。随着半径减小，内部传递距离变短，因此收缩会影响干燥速度。

附录 4 的局部物性为

ρ=760+90C,cp=1850+2150CC+1,k=0.12+0.20CC+1	(18)

D(C,T)=4.2×10−4exp0.30Cexp3850TK	(19)

## 7.3 固定物理位置的映射与结果

题目要求给出固定位置的含水率。因此每个时刻我们先用 ξ=r/R(t) 找到相应的计算位置，再读取该处结果。当 r>R(t) 时，该位置已经在药材之外，工作簿中的单元格留空；“药材表面”一列始终取 ξ=1，随边界移动。

第四问临界时刻为 183931.0374 s（51.0920 h），首个严格达标整分钟为 183960 s（51.1000 h）。

表 6  收缩药材烘干过程的水分浓度（kg/kg）

| 时间/h | 0 cm | 0.5 cm | 1.0 cm | 药材表面 |
| --- | --- | --- | --- | --- |
| 6 | 1.7196 | 1.5377 | 1.0226 | 0.4207 |
| 12 | 0.7377 | 0.6546 | 0.4083 | 0.1669 |
| 18 | 0.4088 | 0.3687 | 0.2397 | 0.0892 |
| 24 | 0.2851 | 0.2611 | 0.1790 | 0.0673 |
| 30 | 0.2263 | 0.2094 | 0.1493 | 0.0595 |
| 36 | 0.1928 | 0.1797 | 0.1317 | 0.0560 |
| 42 | 0.1712 | 0.1604 | 0.1200 | 0.0541 |
| 48 | 0.1561 | 0.1468 | 0.1116 | 0.0530 |
| 烘干临界时刻 51.091955 | 0.1500 | 0.1413 | 0.1081 | 0.0526 |

由附件 2 可知，6 h 时药材半径已经减小到 1.374 cm，此后还将继续减小。可以看到，在表 6 所列时刻，距中心 1.5 cm 和 2 cm 的位置均已位于药材之外。我们只列出 0、0.5、1 cm 和实际表面。完整的 result4.xlsx 仍保留所有间隔为 0.1 cm 的固定位置，并用空白表示实体外位置。

图 4 附件 2 的半径历史及分段线性插值。

图 5 收缩药材在代表时刻的径向含水率；每条曲线右端是当时实际表面。

问题三和问题四的临界时长相差约 6.38 h，但两问同时改变了物性和几何条件。为单独考察收缩的影响，第 8.4 节保持附录 4 物性不变，只把半径固定为初始值进行对照。

# 8 模型检验与灵敏度分析

## 8.1 网格与程序一致性

我们先加密空间网格。第一问两级网格在全部输出节点的最大温度和含水率差分别约为 1.03×10⁻⁶ ℃和 4.57×10⁻⁶ kg/kg；第二问前三小时的最大差分别约为 8.14×10⁻⁷ ℃和 1.49×10⁻⁵ kg/kg。可以看到，网格加密后结果变化很小。因此我们取式（4）的外推值作为最终计算结果。

表 7 两类终点的空间加密结果

| 模型 | 径向区间或外推组合 | 临界时长/h |
| --- | --- | --- |
| 第三问 | 1280 | 57.4881201 |
| 第三问 | 2560 | 57.4772860 |
| 第三问 | 5120 | 57.4747882 |
| 第三问 | 2560/5120 外推 | 57.4739569 |
| 第四问 | 640 | 51.0996704 |
| 第四问 | 1280 | 51.0938837 |
| 第四问 | 640/1280 外推 | 51.0919548 |

由表 7 可知，第三、四问相邻外推组合的终点差分别约为 1.02 s 和 0.98 s。第四问再用单元中心有限体积程序独立计算，1600/3200 单元外推得到 51.0919474 h，表 6 的代表时刻结果与主程序保留四位小数后的结果一致。由此可见，数值计算结果较为稳定。由于观测收敛阶低于 2，外推差只作为误差量级参考。

此外，程序还检验了平衡场保持、边界通量方向、固定半径退化和阈值插值等情况，95 项自动化测试均通过。表 1—6 与数值输出一致，工作簿回读和跨问题比较见附录 A.2。

## 8.2 水分通量平衡

为检查程序有没有漏掉水分，我们再对水分通量作平衡核验。固定半径下，把水分方程在截面上积分，并代入表面边界条件，截面平均含水率满足

C(t)−C(0)=−2hmR00t[Cs(τ)−C∞(τ)]dτ	(20)

移动域中，面积权重变为 2ξ，故定义 C=201Cξdξ。把式（16）在 0≤ξ≤1 上积分，再代入式（17），得到

dCdt=−2hmR(t)[Cs(t)−C∞(t)]	(21)

我们利用前 600 s 的逐秒输出和后续分钟输出，对边界通量进行积分。第三问两级网格的相对最大残差约为 1.21×10⁻⁶，第四问分别约为 1.61×10⁻⁶ 和 1.69×10⁻⁶。残差很小，说明计算结果满足模型中的水分通量平衡。

## 8.3 长期环境与表面系数情景

表 8 第三问长期平台敏感性（同为 640 区间）

| 4 h 后环境设定 | 临界时长/h |
| --- | --- |
| 最后一小时均值 | 57.5413 |
| 最后半小时均值 | 57.5151 |
| 最后观测值保持 | 57.2375 |
| 温度降低、含湿量升高各一个尾段标准差 | 57.8259 |
| 温度升高、含湿量降低各一个尾段标准差 | 57.2587 |

表 8 中各情景采用相同网格，临界时长相差约 0.59 h，明显大于相邻网格外推的差异。可以看出，长期环境设定对终点的影响较大。表中数值是指定情景下的计算结果，不表示统计置信区间。

表 9 第四问表面系数敏感性（同为 320 区间）

| 变化系数 | 基准的 0.8 倍/h | 基准/h | 基准的 1.2 倍/h |
| --- | --- | --- | --- |
| 传质系数 | 51.8009 | 51.1236 | 50.7400 |
| 传热系数 | 51.1418 | 51.1236 | 51.1117 |

由表 9 可以看出，在给定取值范围内，传质系数变化引起的终点偏移明显大于传热系数变化的影响。这说明干燥后期主要受水分迁移控制。0.8 倍和 1.2 倍只是设定的扰动幅度，实际参数还需要实验标定。

## 8.4 收缩处理与固定几何对照

首先保持附件 2 的观测点不变，将分段线性插值改为单调 PCHIP。320 区间计算得到的终点提前约 12.603 s。再把半径观测时刻统一提前或推后 15 min，终点相对同组基准分别改变约 −0.135 h 和 +0.137 h。这两种计算分别考察插值方法和观测时刻的影响。

在附录 4 物性不变、半径固定为 R=R0 的对照模型中，640/1280 区间外推终点为 129.8481 h，而实测收缩模型约为 51.0920 h。半径减小会同时改变式（16）的内部扩散尺度和式（17）的表面尺度，因此对水分迁移影响很大。该对照只用于考察模型中的几何因素，不是对真实收缩效应的独立实验测量。

## 8.5 密度与收缩的相容性

为检验密度关系与收缩假设是否相容，我们暂时把附录 4 的 ρ(C) 看作当前真实湿基密度。干基含水率为 C 时，湿质量是干物质量的 1+C 倍，因此有 ρd=ρ(C)/(1+C)。再把干密度在当前截面上积分，总干物质量为

Md(t)=2πLR(t)201760+90C(ξ,t)1+C(ξ,t)ξdξ	(22)

我们将非均匀含水率场代入式（22）积分。在 1280 区间的临界时刻得到 Md(t*)/Md(0)≈0.890289，偏差约为 10.97%。这说明，若把给定密度看作真实湿基密度，长度不变的实测收缩与干物质守恒不能同时严格成立。该偏差用于检验模型关系，并不表示药材实际损失了相同比例的干物质。

我们在热方程中用附录密度表示经验有效体积热容，在水分方程中按均匀收缩假设确定干密度。这样可以得到当前题设下的有效模型，但密度与几何关系的不相容仍是模型限制。要建立更严格的多组分守恒模型，还需要孔隙率、局部变形或长度变化等数据，现有附件无法确定这些关系。

# 9 模型评价与结论

## 9.1 模型优点

我们用统一的有限体积方法处理常物性、变物性和实测收缩三种情况，得到题设位置的温度和含水率，并以全域最大含水率确定干燥终点。问题四采用移动坐标后，我们可以在固定计算区间内处理收缩边界，同时区分实际表面和固定位置。网格加密、通量平衡和独立程序复算均支持计算结果。

## 9.2 模型缺点及改进方向

模型还有以下不足：一维中截面近似没有考虑端面影响；长期环境由前 4 h 观测延拓得到；气固边界采用有效浓度差并沿用题设表面系数；模型没有计入潜热，经验密度与收缩关系也未严格闭合。现有检验说明数值求解相互一致，物理预测精度仍需要内部含水率观测验证。

后续可补充长期烘房记录和内部温湿度测量，用于检验环境平台和干燥终点；再结合二维模型、吸附平衡关系和几何变形观测，进一步分析端面效应和质量闭合对结果的影响。

## 9.3 结论

综合四个问题的结果我们可以看到，药材先在表层快速失水，随后内部水分缓慢向外迁移；即使温度已经接近均匀，含水率仍可能高于阈值。在上述假设下，固定半径模型和实测收缩模型的临界时长分别为 57.4740 h 和 51.0920 h；若按整分钟停止，则分别取 57.4833 h 和 51.1000 h。相同物性下的固定几何对照说明，收缩是影响预测时长的重要因素。实际应用时，还需要结合长期环境记录和内部含水率测量进行校验。

# AI 工具使用声明

本参赛队在竞赛过程中使用了AI工具，主要用于代码调试、重复性计算检查、文件一致性检查和语言润色，详细使用情况见支撑材料。

本题的核心建模与分析由参赛队主导：队员确定模型框架、假设、参数和边界条件，审定数值结果及其解释，并决定最终提交内容。AI 的任何建议、代码修改或文字均经队员逐项审查、修改和核验。参赛队对模型、代码、数值、参考文献和最终表述承担责任。工具的具体用途与核验方式见支撑材料中的《AI 工具使用详情.pdf》，本节披露不作为参考文献引用。

# 参考文献

[1] 全国大学生数学建模竞赛组委会. 2026 年高教社杯全国大学生数学建模竞赛 A 题：药材的烘干问题及附件[Z]. 2026.

[2] DA SILVA W P, E SILVA C M D P S, GAMA F J A. Estimation of thermo-physical properties of products with cylindrical shape during drying: The coupling between mass and heat[J]. Journal of Food Engineering, 2014, 141: 65-73. DOI: 10.1016/j.jfoodeng.2014.05.010.

[3] ADROVER A, VENDITTI C, BRASIELLO A. A non-isothermal moving-boundary model for continuous and intermittent drying of pears[J]. Foods, 2020, 9(11): 1577. DOI: 10.3390/foods9111577.

# 附录 A 支撑材料与复现说明

支撑材料 ZIP 内的文件列表如下。论文电子版从摘要页开始，不含承诺书和编号专用页。文中四位小数用于题设输出，终点判定始终使用未舍入结果。

AI 工具使用详情.pdf

README_复现说明.md

__init__.py

attachment1.csv

attachment2.csv

audit_all_results.py

build_result4.py

compress_problem2_full_process.py

extend_problem2_delivery.py

final_all_questions_audit.json

flat_support_audit.json

problem1.py

problem1_radial_profiles.png

problem1_time_histories.png

problem2.py

problem2_full_process_audit.json

problem2_radial_profiles.png

problem2_time_histories.png

problem3.py

problem3_radial_profiles.png

problem3_time_histories.png

problem4.py

problem4_independent.py

problem4_independent_validation.json

problem4_production_diagnostics.json

problem4_radial_profiles.png

problem4_radius_history.png

problem4_review_experiments.json

problem4_time_histories.png

problem4_workbook_audit.json

rebuild_delivery_workbooks.py

requirements.txt

restore_full_result2.py

result1.xlsx

result2.xlsx

result2_full_process.npz

result3.xlsx

result4.xlsx

run_problem1.py

run_problem2.py

run_problem3.py

run_problem4.py

test_final_review_regressions.py

test_problem1.py

test_problem2.py

test_problem3.py

test_problem4.py

test_problem4_independent.py

test_result4_workbook.py

verify_problem4.py

verify_problem4_independent.py

## A.1 运行顺序

先在支撑材料根目录安装 requirements.txt 中的依赖。Python 版本须为 3.11 或以上。支撑材料采用单层结构，按下面顺序运行即可：

python -m pip install -r requirements.txt

python run_problem1.py

python run_problem2.py

python run_problem3.py

python run_problem4.py

四个求解脚本在根目录产生 problem1_result.json 至 problem4_result.json，并生成相应图形；不会覆盖已填写的四份结果工作簿。复现所需 CSV 已包含在支撑材料中。

若仅需查看全程第二问 Excel，无须重新求解，在支撑材料根目录执行 python restore_full_result2.py，即可由压缩数组还原 result2_full_process.xlsx。

## A.2 全程输出与数值核验记录

第二问全程逐秒 Excel 约 27.7 MB。为满足支撑材料压缩包不超过 20 MB 的要求，包中以 result2_full_process.npz 无损保存四位小数交付值，并附 Excel 恢复脚本；result2.xlsx 为原前三小时文件。包外另提供的全程 Excel 与恢复文件的两张工作表 XML 逐字节相同。

全程积分沿用第二问的方程、初值和完整节点终态，输出不由舍入后的分钟值插值得到。其前 3 h 与原第二问交付值完全一致，与第三问独立分钟积分的最大四位小数差为 0.0001 kg/kg，差异来自积分分段与舍入边界。

原四份工作簿共回读 700974 个单元格，题设表 1—6 与对应数值输出一致；第二、三问原始分钟序列在前三小时重叠的 3780 个含水率值相同。第二问全程逐秒工作簿另有独立回读记录，见 problem2_full_process_audit.json。

支撑材料根目录保存原四问审计、第四问敏感性和密度诊断、全程逐秒结果审计。程序核验与参赛队人工审查是两类不同记录；AI 工具使用详情仅记录可确认的自动检查，不代替或虚构人工核验。

# 附录 B 核心源程序

下列仅列出问题一至问题四的核心模型代码。运行入口、独立验证、测试程序和工作簿工具等完整源码均直接放在支撑材料根目录，附录不重复展开。附录页数不计入正文限制。

## problem1.py

"""Problem 1 material laws and chamber boundary interpolation."""

from __future__ import annotations

import csv

from dataclasses import dataclass

from pathlib import Path

import numpy as np

from numpy.typing import ArrayLike, NDArray

from scipy.integrate import solve_ivp

from scipy.sparse import diags

@dataclass(frozen=True)

class ChamberHistory:

"""Observed chamber boundary conditions."""

time_s: NDArray[np.float64]

temperature_c: NDArray[np.float64]

moisture_concentration: NDArray[np.float64]

def __post_init__(self) -> None:

time = np.asarray(self.time_s, dtype=float)

temperature = np.asarray(self.temperature_c, dtype=float)

moisture = np.asarray(self.moisture_concentration, dtype=float)

if time.ndim != 1 or temperature.shape != time.shape or moisture.shape != time.shape:

raise ValueError("Chamber history columns must be one-dimensional and equal-sized")

if any(np.any(~np.isfinite(values)) for values in (time, temperature, moisture)):

raise ValueError("Chamber history must contain only finite values")

if time.size < 2 or np.any(np.diff(time) <= 0.0):

raise ValueError("Chamber times must be strictly increasing")

if np.any(moisture < 0.0):

raise ValueError("Chamber moisture concentration must be non-negative")

object.__setattr__(self, "time_s", time)

object.__setattr__(self, "temperature_c", temperature)

object.__setattr__(self, "moisture_concentration", moisture)

@dataclass(frozen=True)

class Problem1Parameters:

radius_m: float = 0.02

initial_temperature_c: float = 28.0

initial_moisture_concentration: float = 2.55

density_kg_m3: float = 820.0

heat_capacity_j_kg_k: float = 2600.0

thermal_conductivity_w_m_k: float = 0.36

heat_transfer_coefficient_w_m2_k: float = 25.0

mass_transfer_coefficient_m_s: float = 8.0e-7

@dataclass(frozen=True)

class Problem1Solution:

time_s: NDArray[np.float64]

radius_m: NDArray[np.float64]

temperature_c: NDArray[np.float64]

moisture_concentration: NDArray[np.float64]

def load_chamber_history_csv(path: str | Path) -> ChamberHistory:

"""Load the three columns supplied in Attachment 1 from a UTF-8 CSV."""

rows: list[tuple[float, float, float]] = []

with Path(path).open("r", encoding="utf-8-sig", newline="") as handle:

reader = csv.DictReader(handle)

required = {"时间", "温度", "水分浓度"}

if reader.fieldnames is None or not required.issubset(reader.fieldnames):

raise ValueError("Attachment 1 must contain 时间, 温度, 水分浓度 columns")

for line_number, row in enumerate(reader, start=2):

try:

rows.append((float(row["时间"]), float(row["温度"]), float(row["水分浓度"])))

except (TypeError, ValueError) as exc:

raise ValueError(f"Invalid numeric value on CSV line {line_number}") from exc

if not rows:

raise ValueError("Attachment 1 contains no data rows")

data = np.asarray(rows, dtype=float)

if not np.all(np.isfinite(data)):

raise ValueError("Attachment 1 contains non-finite values")

return ChamberHistory(data[:, 0], data[:, 1], data[:, 2])

def diffusivity_q1(concentration: ArrayLike) -> NDArray[np.float64]:

"""Return D=7e-9*exp(-0.89/C) in square metres per second."""

values = np.asarray(concentration, dtype=float)

if np.any(values <= 0.0):

raise ValueError("Moisture concentration must be positive")

return 7.0e-9 * np.exp(-0.89 / values)

def interpolate_history(

query_time_s: float,

time_s: ArrayLike,

values: ArrayLike,

) -> float:

"""Linearly interpolate an observed chamber history."""

return float(np.interp(query_time_s, time_s, values))

def nodal_control_volumes(node_count: int, radius_m: float) -> NDArray[np.float64]:

"""Return annular control volumes per unit cylinder length for nodal radii."""

if node_count < 2:

raise ValueError("At least two radial nodes are required")

if radius_m <= 0.0:

raise ValueError("Cylinder radius must be positive")

spacing = radius_m / (node_count - 1)

radii = np.linspace(0.0, radius_m, node_count)

inner = np.maximum(radii - 0.5 * spacing, 0.0)

outer = np.minimum(radii + 0.5 * spacing, radius_m)

return np.pi * (outer**2 - inner**2)

def radial_flux_divergence(

state: ArrayLike,

coefficient: ArrayLike,

radius_m: float,

exchange_coefficient: float,

ambient_value: float,

) -> NDArray[np.float64]:

"""Conservative cylindrical divergence with a convective outer boundary.

``coefficient`` is k for heat or D for moisture.  At the outer surface the

imposed flux is ``-exchange_coefficient * (surface - ambient)``.

"""

values = np.asarray(state, dtype=float)

conductance = np.asarray(coefficient, dtype=float)

if values.ndim != 1 or conductance.shape != values.shape:

raise ValueError("State and coefficient must be one-dimensional and equal-sized")

if np.any(conductance < 0.0) or exchange_coefficient < 0.0:

raise ValueError("Transport coefficients must be non-negative")

node_count = values.size

spacing = radius_m / (node_count - 1)

face_radii = (np.arange(node_count - 1, dtype=float) + 0.5) * spacing

denominator = conductance[:-1] + conductance[1:]

face_coefficient = np.divide(

2.0 * conductance[:-1] * conductance[1:],

denominator,

out=np.zeros_like(denominator),

where=denominator > 0.0,

)

face_gradient = np.diff(values) / spacing

face_rate = 2.0 * np.pi * face_radii * face_coefficient * face_gradient

integrated_rate = np.zeros_like(values)

integrated_rate[:-1] += face_rate

integrated_rate[1:] -= face_rate

surface_flux = -exchange_coefficient * (values[-1] - ambient_value)

integrated_rate[-1] += 2.0 * np.pi * radius_m * surface_flux

return integrated_rate / nodal_control_volumes(node_count, radius_m)

def solve_problem1(

history: ChamberHistory,

radial_intervals: int,

output_times_s: ArrayLike,

*,

parameters: Problem1Parameters = Problem1Parameters(),

relative_tolerance: float = 1.0e-8,

max_step_s: float = 5.0,

) -> Problem1Solution:

"""Solve the fixed-radius preheating model at requested output times."""

if radial_intervals < 2:

raise ValueError("At least two radial intervals are required")

output_times = np.asarray(output_times_s, dtype=float)

if output_times.ndim != 1 or output_times.size == 0:

raise ValueError("Output times must be a non-empty one-dimensional array")

if np.any(np.diff(output_times) <= 0.0):

raise ValueError("Output times must be strictly increasing")

if output_times[0] < history.time_s[0] or output_times[-1] > history.time_s[-1]:

raise ValueError("Output times must lie within the chamber history")

node_count = radial_intervals + 1

radii = np.linspace(0.0, parameters.radius_m, node_count)

sparsity = diags(

[np.ones(node_count - 1), np.ones(node_count), np.ones(node_count - 1)],

offsets=[-1, 0, 1],

shape=(node_count, node_count),

format="csr",

)

thermal_coefficient = np.full(node_count, parameters.thermal_conductivity_w_m_k)

def temperature_rhs(time_s: float, temperature_c: NDArray[np.float64]) -> NDArray[np.float64]:

ambient = interpolate_history(time_s, history.time_s, history.temperature_c)

divergence = radial_flux_divergence(

temperature_c,

thermal_coefficient,

parameters.radius_m,

parameters.heat_transfer_coefficient_w_m2_k,

ambient,

)

return divergence / (parameters.density_kg_m3 * parameters.heat_capacity_j_kg_k)

def log_moisture_rhs(time_s: float, log_concentration: NDArray[np.float64]) -> NDArray[np.float64]:

concentration = np.exp(log_concentration)

ambient = interpolate_history(time_s, history.time_s, history.moisture_concentration)

divergence = radial_flux_divergence(

concentration,

diffusivity_q1(concentration),

parameters.radius_m,

parameters.mass_transfer_coefficient_m_s,

ambient,

)

return divergence / concentration

time_span = (float(history.time_s[0]), float(output_times[-1]))

temperature_result = solve_ivp(

temperature_rhs,

time_span,

np.full(node_count, parameters.initial_temperature_c),

method="BDF",

t_eval=output_times,

rtol=relative_tolerance,

atol=1.0e-9,

max_step=max_step_s,

jac_sparsity=sparsity,

)

moisture_result = solve_ivp(

log_moisture_rhs,

time_span,

np.full(node_count, np.log(parameters.initial_moisture_concentration)),

method="BDF",

t_eval=output_times,

rtol=relative_tolerance,

atol=1.0e-10,

max_step=max_step_s,

jac_sparsity=sparsity,

)

if not temperature_result.success:

raise RuntimeError(f"Temperature solver failed: {temperature_result.message}")

if not moisture_result.success:

raise RuntimeError(f"Moisture solver failed: {moisture_result.message}")

return Problem1Solution(

time_s=output_times,

radius_m=radii,

temperature_c=temperature_result.y.T,

moisture_concentration=np.exp(moisture_result.y.T),

)

def sample_solution(solution: Problem1Solution, radius_cm: ArrayLike) -> Problem1Solution:

"""Interpolate a solution to requested physical radii in centimetres."""

requested_radius_m = np.asarray(radius_cm, dtype=float) * 0.01

if requested_radius_m.ndim != 1 or requested_radius_m.size == 0:

raise ValueError("Requested radii must be a non-empty one-dimensional array")

if np.any(np.diff(requested_radius_m) <= 0.0):

raise ValueError("Requested radii must be strictly increasing")

if requested_radius_m[0] < 0.0 or requested_radius_m[-1] > solution.radius_m[-1]:

raise ValueError("Requested radii lie outside the cylinder")

temperature = np.vstack(

[np.interp(requested_radius_m, solution.radius_m, row) for row in solution.temperature_c]

)

moisture = np.vstack(

[

np.interp(requested_radius_m, solution.radius_m, row)

for row in solution.moisture_concentration

]

)

return Problem1Solution(

time_s=solution.time_s.copy(),

radius_m=requested_radius_m,

temperature_c=temperature,

moisture_concentration=moisture,

)

def richardson_extrapolate_solutions(

coarse: Problem1Solution,

fine: Problem1Solution,

*,

order: int = 2,

) -> Problem1Solution:

"""Extrapolate two solutions whose spatial mesh widths differ by a factor of two."""

if order <= 0:

raise ValueError("Richardson order must be positive")

if (

coarse.temperature_c.shape != fine.temperature_c.shape

or coarse.moisture_concentration.shape

!= fine.moisture_concentration.shape

):

raise ValueError("Solutions must have matching field shapes")

if not np.array_equal(coarse.time_s, fine.time_s) or not np.array_equal(

coarse.radius_m,

fine.radius_m,

):

raise ValueError("Solutions must use identical output times and radii")

denominator = 2.0**order - 1.0

return Problem1Solution(

time_s=fine.time_s.copy(),

radius_m=fine.radius_m.copy(),

temperature_c=fine.temperature_c

+ (fine.temperature_c - coarse.temperature_c) / denominator,

moisture_concentration=fine.moisture_concentration

+ (fine.moisture_concentration - coarse.moisture_concentration) / denominator,

)

def result_payload(solution: Problem1Solution) -> dict[str, list]:

"""Convert a sampled solution to the numeric schema used by result1.xlsx."""

if np.any(~np.isfinite(solution.time_s)) or not np.allclose(solution.time_s, np.round(solution.time_s), atol=1.0e-10, rtol=0.0):

raise ValueError("Workbook output times must be whole seconds")

return {

"time_s": np.round(solution.time_s).astype(int).tolist(),

"radius_cm": np.round(solution.radius_m * 100.0, 10).tolist(),

"temperature_c": np.round(solution.temperature_c, 4).tolist(),

"moisture_concentration": np.round(solution.moisture_concentration, 4).tolist(),

}

## problem2.py

"""Coupled variable-property model for Problem 2."""

from __future__ import annotations

from collections.abc import Callable

import numpy as np

from numpy.typing import ArrayLike, NDArray

from scipy.integrate import solve_ivp

from scipy.sparse import bmat, diags

from problem1 import (

ChamberHistory,

Problem1Parameters,

Problem1Solution,

interpolate_history,

radial_flux_divergence,

sample_solution,

)

ScalarLaw = Callable[[NDArray[np.float64]], NDArray[np.float64]]

CoupledLaw = Callable[

[NDArray[np.float64], NDArray[np.float64]],

NDArray[np.float64],

]

def _concentration_array(concentration: ArrayLike) -> NDArray[np.float64]:

values = np.asarray(concentration, dtype=float)

if np.any(~np.isfinite(values)) or np.any(values < 0.0):

raise ValueError("Moisture concentration must be finite and non-negative")

return values

def density_q2(concentration: ArrayLike) -> NDArray[np.float64]:

"""Return Appendix 3 density in kilograms per cubic metre."""

values = _concentration_array(concentration)

return 650.0 + 128.0 * values

def heat_capacity_q2(concentration: ArrayLike) -> NDArray[np.float64]:

"""Return Appendix 3 specific heat capacity in joules per kilogram-kelvin."""

values = _concentration_array(concentration)

return 1450.0 + 2736.0 * values / (values + 1.0)

def thermal_conductivity_q2(concentration: ArrayLike) -> NDArray[np.float64]:

"""Return Appendix 3 thermal conductivity in watts per metre-kelvin."""

values = _concentration_array(concentration)

return 0.21 + 0.38 * values / (values + 1.0)

def diffusivity_q2(

concentration: ArrayLike,

temperature_c: ArrayLike,

) -> NDArray[np.float64]:

"""Return Appendix 3 moisture diffusivity using absolute temperature."""

moisture = _concentration_array(concentration)

temperature_k = np.asarray(temperature_c, dtype=float) + 273.15

if np.any(~np.isfinite(temperature_k)) or np.any(temperature_k <= 0.0):

raise ValueError("Temperature must be finite and above absolute zero")

if np.any(moisture <= 0.0):

raise ValueError("Moisture concentration must be positive for diffusivity")

moisture, temperature_k = np.broadcast_arrays(moisture, temperature_k)

return 2.4e-3 * np.exp(-0.45 / moisture) * np.exp(-3850.0 / temperature_k)

def solve_variable_property_fixed_cylinder(

history: ChamberHistory,

radial_intervals: int,

output_times_s: ArrayLike,

*,

density_law: ScalarLaw,

heat_capacity_law: ScalarLaw,

conductivity_law: ScalarLaw,

diffusivity_law: CoupledLaw,

parameters: Problem1Parameters = Problem1Parameters(),

relative_tolerance: float = 1.0e-8,

max_step_s: float = 5.0,

maximum_moisture_threshold: float | None = None,

initial_time_s: float | None = None,

initial_temperature_c: ArrayLike | None = None,

initial_moisture_concentration: ArrayLike | None = None,

) -> Problem1Solution:

"""Solve coupled heat and moisture transport on a fixed cylindrical radius."""

if radial_intervals < 2:

raise ValueError("At least two radial intervals are required")

output_times = np.asarray(output_times_s, dtype=float)

if output_times.ndim != 1 or output_times.size == 0:

raise ValueError("Output times must be a non-empty one-dimensional array")

if np.any(np.diff(output_times) <= 0.0):

raise ValueError("Output times must be strictly increasing")

start_time = history.time_s[0] if initial_time_s is None else float(initial_time_s)

if (

not np.isfinite(start_time)

or start_time < history.time_s[0]

or start_time > output_times[0]

or output_times[-1] > history.time_s[-1]

):

raise ValueError("Output times must lie within the chamber history")

if maximum_moisture_threshold is not None and (

not np.isfinite(maximum_moisture_threshold)

or maximum_moisture_threshold <= 0.0

or maximum_moisture_threshold >= parameters.initial_moisture_concentration

):

raise ValueError("Maximum moisture threshold must lie between zero and the initial value")

node_count = radial_intervals + 1

radii = np.linspace(0.0, parameters.radius_m, node_count)

tridiagonal = diags(

[np.ones(node_count - 1), np.ones(node_count), np.ones(node_count - 1)],

offsets=[-1, 0, 1],

shape=(node_count, node_count),

format="csr",

)

sparsity = bmat(

[[tridiagonal, tridiagonal], [tridiagonal, tridiagonal]],

format="csr",

)

def coupled_rhs(time_s: float, state: NDArray[np.float64]) -> NDArray[np.float64]:

temperature_c = state[:node_count]

concentration = np.exp(state[node_count:])

density = np.asarray(density_law(concentration), dtype=float)

heat_capacity = np.asarray(heat_capacity_law(concentration), dtype=float)

conductivity = np.asarray(conductivity_law(concentration), dtype=float)

diffusivity = np.asarray(

diffusivity_law(concentration, temperature_c),

dtype=float,

)

expected_shape = concentration.shape

for name, values in (

("density", density),

("heat capacity", heat_capacity),

("thermal conductivity", conductivity),

("diffusivity", diffusivity),

):

if values.shape != expected_shape or np.any(~np.isfinite(values)) or np.any(values <= 0.0):

raise ValueError(f"{name} law must return finite positive nodal values")

ambient_temperature = interpolate_history(

time_s,

history.time_s,

history.temperature_c,

)

temperature_rate = radial_flux_divergence(

temperature_c,

conductivity,

parameters.radius_m,

parameters.heat_transfer_coefficient_w_m2_k,

ambient_temperature,

) / (density * heat_capacity)

ambient_moisture = interpolate_history(

time_s,

history.time_s,

history.moisture_concentration,

)

concentration_rate = radial_flux_divergence(

concentration,

diffusivity,

parameters.radius_m,

parameters.mass_transfer_coefficient_m_s,

ambient_moisture,

)

log_concentration_rate = concentration_rate / concentration

return np.concatenate([temperature_rate, log_concentration_rate])

supplied_initial_fields = (

initial_temperature_c is not None,

initial_moisture_concentration is not None,

)

if any(supplied_initial_fields) and not all(supplied_initial_fields):

raise ValueError("Both initial fields must be supplied together")

if all(supplied_initial_fields):

temperature_initial = np.asarray(initial_temperature_c, dtype=float)

moisture_initial = np.asarray(initial_moisture_concentration, dtype=float)

if (

temperature_initial.shape != (node_count,)

or moisture_initial.shape != (node_count,)

or np.any(~np.isfinite(temperature_initial))

or np.any(~np.isfinite(moisture_initial))

or np.any(moisture_initial <= 0.0)

):

raise ValueError("Initial fields must be finite positive nodal arrays")

else:

temperature_initial = np.full(node_count, parameters.initial_temperature_c)

moisture_initial = np.full(

node_count,

parameters.initial_moisture_concentration,

)

initial_state = np.concatenate(

[temperature_initial, np.log(moisture_initial)]

)

absolute_tolerance = np.concatenate(

[np.full(node_count, 1.0e-9), np.full(node_count, 1.0e-10)]

)

threshold_event = None

if maximum_moisture_threshold is not None:

def threshold_event(time_s: float, state: NDArray[np.float64]) -> float:

del time_s

return float(

np.max(np.exp(state[node_count:])) - maximum_moisture_threshold

)

threshold_event.terminal = True

threshold_event.direction = -1.0

result = solve_ivp(

coupled_rhs,

(start_time, float(output_times[-1])),

initial_state,

method="BDF",

t_eval=output_times,

rtol=relative_tolerance,

atol=absolute_tolerance,

max_step=max_step_s,

jac_sparsity=sparsity,

events=threshold_event,

)

if not result.success:

raise RuntimeError(f"Coupled solver failed: {result.message}")

result_time = result.t

result_state = result.y

if maximum_moisture_threshold is not None:

if not result.t_events or result.t_events[0].size == 0:

raise RuntimeError("Maximum moisture threshold was not reached")

event_time = float(result.t_events[0][0])

event_state = result.y_events[0][0]

if result_time.size and np.isclose(

result_time[-1],

event_time,

rtol=0.0,

atol=1.0e-9,

):

result_time = result_time.copy()

result_state = result_state.copy()

result_time[-1] = event_time

result_state[:, -1] = event_state

else:

result_time = np.append(result_time, event_time)

result_state = np.column_stack([result_state, event_state])

return Problem1Solution(

time_s=result_time,

radius_m=radii,

temperature_c=result_state[:node_count].T,

moisture_concentration=np.exp(result_state[node_count:].T),

)

def solve_problem2(

history: ChamberHistory,

radial_intervals: int,

output_times_s: ArrayLike,

*,

parameters: Problem1Parameters = Problem1Parameters(),

relative_tolerance: float = 1.0e-8,

max_step_s: float = 5.0,

maximum_moisture_threshold: float | None = None,

initial_time_s: float | None = None,

initial_temperature_c: ArrayLike | None = None,

initial_moisture_concentration: ArrayLike | None = None,

) -> Problem1Solution:

"""Solve Problem 2 with the four Appendix 3 material-property laws."""

return solve_variable_property_fixed_cylinder(

history,

radial_intervals,

output_times_s,

density_law=density_q2,

heat_capacity_law=heat_capacity_q2,

conductivity_law=thermal_conductivity_q2,

diffusivity_law=diffusivity_q2,

parameters=parameters,

relative_tolerance=relative_tolerance,

max_step_s=max_step_s,

maximum_moisture_threshold=maximum_moisture_threshold,

initial_time_s=initial_time_s,

initial_temperature_c=initial_temperature_c,

initial_moisture_concentration=initial_moisture_concentration,

)

def solve_problem2_sampled_in_chunks(

history: ChamberHistory,

radial_intervals: int,

output_times_s: ArrayLike,

sample_radius_cm: ArrayLike,

*,

chunk_duration_s: float = 600.0,

parameters: Problem1Parameters = Problem1Parameters(),

relative_tolerance: float = 1.0e-8,

max_step_s: float = 5.0,

) -> Problem1Solution:

"""Solve Problem 2 in bounded-memory chunks and retain sampled radii only."""

output_times = np.asarray(output_times_s, dtype=float)

if output_times.ndim != 1 or output_times.size == 0:

raise ValueError("Output times must be a non-empty one-dimensional array")

if np.any(~np.isfinite(output_times)) or np.any(np.diff(output_times) <= 0.0):

raise ValueError("Output times must be finite and strictly increasing")

if (

output_times[0] < history.time_s[0]

or output_times[-1] > history.time_s[-1]

):

raise ValueError("Output times must lie within the chamber history")

if not np.isfinite(chunk_duration_s) or chunk_duration_s <= 0.0:

raise ValueError("Chunk duration must be finite and positive")

requested_radius_cm = np.asarray(sample_radius_cm, dtype=float)

node_count = radial_intervals + 1

nodal_radius = np.linspace(0.0, parameters.radius_m, node_count)

initial_solution = Problem1Solution(

time_s=np.array([history.time_s[0]]),

radius_m=nodal_radius,

temperature_c=np.full((1, node_count), parameters.initial_temperature_c),

moisture_concentration=np.full(

(1, node_count),

parameters.initial_moisture_concentration,

),

)

sampled_initial = sample_solution(initial_solution, requested_radius_cm)

sampled_temperature = np.empty(

(output_times.size, sampled_initial.radius_m.size),

dtype=float,

)

sampled_moisture = np.empty_like(sampled_temperature)

next_output_index = 0

current_time = float(history.time_s[0])

current_temperature: NDArray[np.float64] | None = None

current_moisture: NDArray[np.float64] | None = None

if np.isclose(output_times[0], current_time, rtol=0.0, atol=1.0e-12):

sampled_temperature[0] = sampled_initial.temperature_c[0]

sampled_moisture[0] = sampled_initial.moisture_concentration[0]

next_output_index = 1

final_time = float(output_times[-1])

while current_time < final_time:

chunk_end = min(current_time + chunk_duration_s, final_time)

chunk_stop = int(np.searchsorted(output_times, chunk_end, side="right"))

requested_times = output_times[next_output_index:chunk_stop]

if requested_times.size and requested_times[0] <= current_time:

raise RuntimeError("Chunked output indexing did not advance")

append_chunk_end = (

requested_times.size == 0

or not np.isclose(requested_times[-1], chunk_end, rtol=0.0, atol=1.0e-12)

)

integration_times = (

np.append(requested_times, chunk_end)

if append_chunk_end

else requested_times

)

chunk_solution = solve_problem2(

history,

radial_intervals=radial_intervals,

output_times_s=integration_times,

parameters=parameters,

relative_tolerance=relative_tolerance,

max_step_s=max_step_s,

initial_time_s=None if current_temperature is None else current_time,

initial_temperature_c=current_temperature,

initial_moisture_concentration=current_moisture,

)

sampled_chunk = sample_solution(chunk_solution, requested_radius_cm)

retained_count = requested_times.size

if retained_count:

sampled_temperature[next_output_index:chunk_stop] = (

sampled_chunk.temperature_c[:retained_count]

)

sampled_moisture[next_output_index:chunk_stop] = (

sampled_chunk.moisture_concentration[:retained_count]

)

current_temperature = chunk_solution.temperature_c[-1].copy()

current_moisture = chunk_solution.moisture_concentration[-1].copy()

current_time = chunk_end

next_output_index = chunk_stop

if next_output_index != output_times.size:

raise RuntimeError("Chunked solver did not populate every requested output time")

return Problem1Solution(

time_s=output_times.copy(),

radius_m=sampled_initial.radius_m,

temperature_c=sampled_temperature,

moisture_concentration=sampled_moisture,

)

## problem3.py

"""Fixed-radius drying endpoint model for Problem 3."""

from __future__ import annotations

from dataclasses import dataclass

import numpy as np

from scipy.integrate import cumulative_simpson

from problem1 import (

ChamberHistory,

Problem1Parameters,

Problem1Solution,

nodal_control_volumes,

)

from problem2 import solve_problem2

@dataclass(frozen=True)

class Problem3Result:

"""Problem 3 solution together with its unrounded drying endpoint."""

solution: Problem1Solution

drying_time_s: float

strict_completion_time_s: float | None

maximum_moisture: float

maximum_radius_m: float

plateau_temperature_c: float

plateau_moisture_concentration: float

threshold: float

output_interval_s: float

def extend_chamber_history_to_plateau(

history: ChamberHistory,

end_time_s: float,

*,

averaging_window_s: float = 3600.0,

transition_s: float = 60.0,

plateau_temperature_c: float | None = None,

plateau_moisture_concentration: float | None = None,

) -> ChamberHistory:

"""Extend Attachment 1 with a short transition to a constant tail plateau."""

if not np.isfinite(end_time_s) or end_time_s <= history.time_s[-1]:

raise ValueError("Extension end time must be later than the observed history")

if not np.isfinite(averaging_window_s) or averaging_window_s <= 0.0:

raise ValueError("Averaging window must be positive")

if not np.isfinite(transition_s) or transition_s <= 0.0:

raise ValueError("Plateau transition must be positive")

tail = history.time_s >= history.time_s[-1] - averaging_window_s

if np.count_nonzero(tail) < 2:

raise ValueError("Averaging window must contain at least two observations")

temperature = (

float(np.mean(history.temperature_c[tail]))

if plateau_temperature_c is None

else float(plateau_temperature_c)

)

moisture = (

float(np.mean(history.moisture_concentration[tail]))

if plateau_moisture_concentration is None

else float(plateau_moisture_concentration)

)

if not np.isfinite(temperature):

raise ValueError("Plateau temperature must be finite")

if not np.isfinite(moisture) or moisture < 0.0:

raise ValueError("Plateau moisture concentration must be finite and non-negative")

transition_time = min(history.time_s[-1] + transition_s, end_time_s)

appended_times = [transition_time]

if end_time_s > transition_time:

appended_times.append(end_time_s)

return ChamberHistory(

time_s=np.concatenate([history.time_s, np.asarray(appended_times)]),

temperature_c=np.concatenate(

[history.temperature_c, np.full(len(appended_times), temperature)]

),

moisture_concentration=np.concatenate(

[history.moisture_concentration, np.full(len(appended_times), moisture)]

),

)

def solve_problem3(

history: ChamberHistory,

radial_intervals: int,

horizon_s: float,

*,

output_interval_s: float = 60.0,

threshold: float = 0.15,

averaging_window_s: float = 3600.0,

transition_s: float = 60.0,

plateau_temperature_c: float | None = None,

plateau_moisture_concentration: float | None = None,

parameters: Problem1Parameters = Problem1Parameters(),

relative_tolerance: float = 1.0e-8,

max_step_s: float = 60.0,

) -> Problem3Result:

"""Continue the Problem 2 model until the full field reaches the threshold."""

if not np.isfinite(output_interval_s) or output_interval_s <= 0.0:

raise ValueError("Output interval must be positive")

extended = extend_chamber_history_to_plateau(

history,

horizon_s,

averaging_window_s=averaging_window_s,

transition_s=transition_s,

plateau_temperature_c=plateau_temperature_c,

plateau_moisture_concentration=plateau_moisture_concentration,

)

regular_times = np.arange(

output_interval_s,

horizon_s + 0.5 * output_interval_s,

output_interval_s,

)

regular_times = regular_times[regular_times <= horizon_s]

if regular_times.size == 0 or regular_times[-1] < horizon_s:

output_times = np.append(regular_times, horizon_s)

else:

output_times = regular_times

solution = solve_problem2(

extended,

radial_intervals,

output_times,

parameters=parameters,

relative_tolerance=relative_tolerance,

max_step_s=max_step_s,

maximum_moisture_threshold=threshold,

)

terminal_moisture = solution.moisture_concentration[-1]

maximum_index = int(np.argmax(terminal_moisture))

return Problem3Result(

solution=solution,

drying_time_s=float(solution.time_s[-1]),

strict_completion_time_s=None,

maximum_moisture=float(terminal_moisture[maximum_index]),

maximum_radius_m=float(solution.radius_m[maximum_index]),

plateau_temperature_c=float(extended.temperature_c[-1]),

plateau_moisture_concentration=float(extended.moisture_concentration[-1]),

threshold=float(threshold),

output_interval_s=float(output_interval_s),

)

def richardson_extrapolate_drying_time(

coarse_time_s: float,

fine_time_s: float,

*,

order: int = 2,

) -> float:

"""Extrapolate drying times from meshes whose spacing ratio is two."""

if order <= 0:

raise ValueError("Richardson order must be positive")

if not np.isfinite(coarse_time_s) or not np.isfinite(fine_time_s):

raise ValueError("Drying times must be finite")

return float(fine_time_s + (fine_time_s - coarse_time_s) / (2.0**order - 1.0))

def validate_radially_nonincreasing_moisture(

solution: Problem1Solution,

*,

tolerance: float = 1.0e-10,

) -> float:

"""Require every stored profile to be wettest at the centre and nonincreasing."""

if not np.isfinite(tolerance) or tolerance < 0.0:

raise ValueError("Radial monotonicity tolerance must be finite and non-negative")

if solution.radius_m.size < 2:

raise ValueError("Radial monotonicity requires at least two positions")

maximum_outward_increase = float(

np.max(np.diff(solution.moisture_concentration, axis=1))

)

if maximum_outward_increase > tolerance:

raise ValueError(

"Moisture profile contains an outward increase; centre-wettest claim failed"

)

return maximum_outward_increase

def moisture_balance_diagnostics(

solution: Problem1Solution,

history: ChamberHistory,

*,

parameters: Problem1Parameters = Problem1Parameters(),

) -> dict[str, float]:

"""Compare total moisture change with integrated convective surface flux."""

if not np.isclose(solution.time_s[0], history.time_s[0], atol=1.0e-12, rtol=0.0):

raise ValueError("Moisture balance requires the initial-time solution row")

if solution.time_s[-1] > history.time_s[-1]:

raise ValueError("Moisture balance times must lie within the chamber history")

if not np.isclose(solution.radius_m[-1], parameters.radius_m, atol=1.0e-12, rtol=0.0):

raise ValueError("Solution radius and model radius must match")

volumes = nodal_control_volumes(solution.radius_m.size, parameters.radius_m)

mean_moisture = solution.moisture_concentration @ volumes / np.sum(volumes)

ambient_moisture = np.interp(

solution.time_s,

history.time_s,

history.moisture_concentration,

)

mean_rate_from_surface = (

-2.0

* parameters.mass_transfer_coefficient_m_s

/ parameters.radius_m

* (solution.moisture_concentration[:, -1] - ambient_moisture)

)

cumulative_surface_change = cumulative_simpson(

mean_rate_from_surface,

x=solution.time_s,

initial=0.0,

)

residual = (mean_moisture - mean_moisture[0]) - cumulative_surface_change

total_change = abs(float(mean_moisture[-1] - mean_moisture[0]))

maximum_residual = float(np.max(np.abs(residual)))

return {

"maximum_absolute_residual": maximum_residual,

"final_residual": float(residual[-1]),

"relative_max_residual": maximum_residual / max(total_change, 1.0e-15),

"total_mean_moisture_change": float(

mean_moisture[-1] - mean_moisture[0]

),

"integrated_surface_change": float(cumulative_surface_change[-1]),

}

def truncate_solution_at_threshold(

solution: Problem1Solution,

*,

threshold: float,

output_interval_s: float,

) -> Problem1Solution:

"""Insert the threshold crossing and retain the first later strict regular row."""

maximum = np.max(solution.moisture_concentration, axis=1)

crossing_indices = np.flatnonzero(maximum <= threshold)

if crossing_indices.size == 0 or crossing_indices[0] == 0:

raise ValueError("Solution must bracket the first moisture threshold crossing")

upper = int(crossing_indices[0])

lower = upper - 1

# Interpolate each nodal field, then take the last nodal crossing. Merely

# interpolating the two maxima is wrong when the wettest node changes.

above = solution.moisture_concentration[lower] > threshold

nodal_drop = solution.moisture_concentration[lower, above] - solution.moisture_concentration[upper, above]

fraction = float(np.max((solution.moisture_concentration[lower, above] - threshold) / nodal_drop))

crossing_time = solution.time_s[lower] + fraction * (

solution.time_s[upper] - solution.time_s[lower]

)

crossing_temperature = solution.temperature_c[lower] + fraction * (

solution.temperature_c[upper] - solution.temperature_c[lower]

)

crossing_moisture = solution.moisture_concentration[lower] + fraction * (

solution.moisture_concentration[upper]

- solution.moisture_concentration[lower]

)

on_regular_interval = np.isclose(

np.mod(solution.time_s, output_interval_s),

0.0,

atol=1.0e-8,

)

regular_before = on_regular_interval & (

solution.time_s < crossing_time - 1.0e-8

)

strict_candidates = np.flatnonzero(

on_regular_interval

& (solution.time_s > crossing_time + 1.0e-8)

& (maximum < threshold)

)

if strict_candidates.size == 0:

raise ValueError("Solution must include a later regular row strictly below threshold")

strict_index = int(strict_candidates[0])

return Problem1Solution(

time_s=np.concatenate(

[solution.time_s[regular_before], [crossing_time, solution.time_s[strict_index]]]

),

radius_m=solution.radius_m.copy(),

temperature_c=np.vstack(

[

solution.temperature_c[regular_before],

crossing_temperature,

solution.temperature_c[strict_index],

]

),

moisture_concentration=np.vstack(

[

solution.moisture_concentration[regular_before],

crossing_moisture,

solution.moisture_concentration[strict_index],

]

),

)

def problem3_payload(result: Problem3Result) -> dict[str, object]:

"""Convert a sampled Problem 3 result to the result3.xlsx data schema."""

solution = result.solution

if solution.time_s.size == 0 or np.any(np.diff(solution.time_s) <= 0.0):

raise ValueError("Problem 3 output times must be non-empty and increasing")

if result.strict_completion_time_s is None:

raise ValueError("Strict completion time requires a verified post-threshold row")

if not np.isclose(

solution.time_s[-1],

result.strict_completion_time_s,

rtol=0.0,

atol=1.0e-8,

):

raise ValueError("The final solution row must be the strict completion time")

critical_indices = np.flatnonzero(

np.isclose(

solution.time_s,

result.drying_time_s,

rtol=0.0,

atol=1.0e-8,

)

)

if critical_indices.size != 1:

raise ValueError("The solution must contain exactly one critical threshold row")

critical_index = int(critical_indices[0])

regular_rows = np.ones(solution.time_s.size, dtype=bool)

regular_rows[critical_index] = False

if not np.allclose(

np.mod(solution.time_s[regular_rows], result.output_interval_s),

0.0,

atol=1.0e-8,

):

raise ValueError("All non-critical output rows must lie on the regular interval")

critical_maximum = float(

np.max(solution.moisture_concentration[critical_index])

)

strict_maximum = float(np.max(solution.moisture_concentration[-1]))

if not np.isclose(critical_maximum, result.threshold, atol=1.0e-8, rtol=0.0):

raise ValueError("Critical maximum moisture must equal the unrounded threshold")

if not strict_maximum < result.threshold:

raise ValueError("Strict completion maximum moisture must be below threshold")

time_values: list[int | float] = []

for value in solution.time_s:

rounded_integer = round(float(value))

if np.isclose(value, rounded_integer, rtol=0.0, atol=1.0e-9):

time_values.append(int(rounded_integer))

else:

time_values.append(round(float(value), 6))

return {

"time_s": time_values,

"radius_cm": np.round(solution.radius_m * 100.0, 10).tolist(),

"moisture_concentration": np.round(

solution.moisture_concentration,

4,

).tolist(),

"drying_time_s": round(result.drying_time_s, 6),

"drying_time_h": round(result.drying_time_s / 3600.0, 10),

"strict_completion_time_s": round(result.strict_completion_time_s, 6),

"strict_completion_time_h": round(

result.strict_completion_time_s / 3600.0,

10,

),

"threshold": result.threshold,

"critical_maximum_moisture_unrounded": critical_maximum,

"strict_completion_maximum_moisture_unrounded": strict_maximum,

"maximum_radius_m": result.maximum_radius_m,

"plateau_temperature_c": result.plateau_temperature_c,

"plateau_moisture_concentration": result.plateau_moisture_concentration,

}

## problem4.py

"""Moving-radius drying model for Problem 4.

The measured radius history is used as a prescribed moving boundary.  The

radial coordinate is mapped to ``xi=r/R(t)``; for dry-basis concentration the

solid motion then cancels the grid velocity and leaves a diffusion equation on

the fixed interval ``0 <= xi <= 1`` under spatially uniform dry-solid density.

Appendix 4 density is used as an effective thermal-storage law; compatibility

with a literal wet-bulk-density interpretation is audited separately.

"""

from __future__ import annotations

import csv

from dataclasses import dataclass, field

from pathlib import Path

import numpy as np

from numpy.typing import ArrayLike, NDArray

from scipy.integrate import cumulative_simpson, solve_ivp

from scipy.interpolate import PchipInterpolator

from scipy.sparse import bmat, diags

from problem1 import (

ChamberHistory,

Problem1Parameters,

interpolate_history,

nodal_control_volumes,

radial_flux_divergence,

)

@dataclass(frozen=True)

class RadiusHistory:

"""Measured radius history in seconds and centimetres converted to metres."""

time_s: NDArray[np.float64]

radius_m: NDArray[np.float64]

interpolation: str = "linear"

_pchip: PchipInterpolator | None = field(init=False, repr=False, compare=False)

def __post_init__(self) -> None:

time = np.asarray(self.time_s, dtype=float)

radius = np.asarray(self.radius_m, dtype=float)

if time.ndim != 1 or radius.shape != time.shape or time.size < 2:

raise ValueError("Radius history columns must be one-dimensional and equal-sized")

if np.any(~np.isfinite(time)) or np.any(~np.isfinite(radius)):

raise ValueError("Radius history must be finite")

if np.any(np.diff(time) <= 0.0):

raise ValueError("Radius times must be strictly increasing")

if np.any(radius <= 0.0) or np.any(np.diff(radius) > 1.0e-12):

raise ValueError("Radius must remain positive and non-increasing")

if self.interpolation not in ("linear", "pchip"):

raise ValueError("Radius interpolation must be linear or pchip")

object.__setattr__(self, "time_s", time)

object.__setattr__(self, "radius_m", radius)

object.__setattr__(self, "_pchip", PchipInterpolator(time, radius) if self.interpolation == "pchip" else None)

@dataclass(frozen=True)

class Problem4Solution:

"""Full solution on the dimensionless moving-boundary coordinate."""

time_s: NDArray[np.float64]

xi: NDArray[np.float64]

temperature_c: NDArray[np.float64]

moisture_concentration: NDArray[np.float64]

def load_radius_history_csv(path: str | Path) -> RadiusHistory:

"""Load Attachment 2 exported as UTF-8 CSV."""

rows: list[tuple[float, float]] = []

with Path(path).open("r", encoding="utf-8-sig", newline="") as handle:

reader = csv.DictReader(handle)

if reader.fieldnames is None:

raise ValueError("Attachment 2 has no header")

names = set(reader.fieldnames)

time_name = next((name for name in ("时间", "time", "time_s", "t") if name in names), None)

radius_name = next((name for name in ("半径", "radius", "radius_cm", "R") if name in names), None)

# The official workbook may be exported with a locale-specific or

# mojibake header.  Attachment 2 has exactly two columns, so a

# positional fallback is unambiguous and keeps the data auditable.

if time_name is None or radius_name is None:

time_name, radius_name = reader.fieldnames[:2]

for line_number, row in enumerate(reader, start=2):

try:

rows.append((float(row[time_name]), float(row[radius_name])))

except (TypeError, ValueError) as exc:

raise ValueError(f"Invalid numeric value on CSV line {line_number}") from exc

if not rows:

raise ValueError("Attachment 2 contains no data rows")

values = np.asarray(rows, dtype=float)

return RadiusHistory(values[:, 0], values[:, 1] * 0.01)

def interpolate_radius(time_s: ArrayLike, history: RadiusHistory) -> NDArray[np.float64]:

"""Interpolate radius with the selected method and hold the measured tail."""

query = np.asarray(time_s, dtype=float)

if np.any(~np.isfinite(query)) or np.any(query < history.time_s[0]):

raise ValueError("Requested radius time precedes Attachment 2")

if history._pchip is not None:

return history._pchip(np.minimum(query, history.time_s[-1]))

return np.interp(query, history.time_s, history.radius_m)

def _concentration_array(concentration: ArrayLike) -> NDArray[np.float64]:

values = np.asarray(concentration, dtype=float)

if np.any(~np.isfinite(values)) or np.any(values < 0.0):

raise ValueError("Moisture concentration must be finite and non-negative")

return values

def density_q4(concentration: ArrayLike) -> NDArray[np.float64]:

values = _concentration_array(concentration)

return 760.0 + 90.0 * values

def heat_capacity_q4(concentration: ArrayLike) -> NDArray[np.float64]:

values = _concentration_array(concentration)

return 1850.0 + 2150.0 * values / (values + 1.0)

def thermal_conductivity_q4(concentration: ArrayLike) -> NDArray[np.float64]:

values = _concentration_array(concentration)

return 0.12 + 0.20 * values / (values + 1.0)

def diffusivity_q4(

concentration: ArrayLike,

temperature_c: ArrayLike,

) -> NDArray[np.float64]:

moisture = _concentration_array(concentration)

temperature_k = np.asarray(temperature_c, dtype=float) + 273.15

if np.any(~np.isfinite(temperature_k)) or np.any(temperature_k <= 0.0):

raise ValueError("Temperature must be finite and above absolute zero")

if np.any(moisture <= 0.0):

raise ValueError("Moisture concentration must be positive for diffusivity")

moisture, temperature_k = np.broadcast_arrays(moisture, temperature_k)

return 4.2e-4 * np.exp(-0.30 / moisture) * np.exp(-3850.0 / temperature_k)

def solve_problem4(

history: ChamberHistory,

radius_history: RadiusHistory,

radial_intervals: int,

output_times_s: ArrayLike,

*,

parameters: Problem1Parameters = Problem1Parameters(),

relative_tolerance: float = 2.0e-8,

max_step_s: float = 60.0,

maximum_moisture_threshold: float | None = None,

initial_time_s: float | None = None,

initial_temperature_c: ArrayLike | None = None,

initial_moisture_concentration: ArrayLike | None = None,

) -> Problem4Solution:

"""Integrate the coupled Appendix 4 equations on ``xi=r/R(t)``."""

if radial_intervals < 2:

raise ValueError("At least two radial intervals are required")

output_times = np.asarray(output_times_s, dtype=float)

if output_times.ndim != 1 or output_times.size == 0 or np.any(~np.isfinite(output_times)):

raise ValueError("Output times must be a non-empty finite array")

if np.any(np.diff(output_times) <= 0.0):

raise ValueError("Output times must be strictly increasing")

start_time = history.time_s[0] if initial_time_s is None else float(initial_time_s)

if not np.isfinite(start_time) or start_time < max(history.time_s[0], radius_history.time_s[0]) or start_time > output_times[0]:

raise ValueError("Initial time must not be after the first output time")

if output_times[-1] > history.time_s[-1]:

raise ValueError("Output times exceed the supplied chamber history")

if maximum_moisture_threshold is not None and not (

0.0 < maximum_moisture_threshold < parameters.initial_moisture_concentration

):

raise ValueError("Maximum moisture threshold must lie between zero and the initial value")

node_count = radial_intervals + 1

xi = np.linspace(0.0, 1.0, node_count)

tri = diags(

[np.ones(node_count - 1), np.ones(node_count), np.ones(node_count - 1)],

offsets=[-1, 0, 1], shape=(node_count, node_count), format="csr",

)

sparsity = bmat([[tri, tri], [tri, tri]], format="csr")

supplied_initial = (initial_temperature_c is not None, initial_moisture_concentration is not None)

if any(supplied_initial) and not all(supplied_initial):

raise ValueError("Both initial fields must be supplied together")

if all(supplied_initial):

temperature_initial = np.asarray(initial_temperature_c, dtype=float)

moisture_initial = np.asarray(initial_moisture_concentration, dtype=float)

if temperature_initial.shape != (node_count,) or moisture_initial.shape != (node_count,):

raise ValueError("Initial fields have the wrong nodal shape")

if np.any(~np.isfinite(temperature_initial)) or np.any(~np.isfinite(moisture_initial)) or np.any(moisture_initial <= 0.0):

raise ValueError("Initial fields must be finite and moisture must be positive")

else:

temperature_initial = np.full(node_count, parameters.initial_temperature_c)

moisture_initial = np.full(node_count, parameters.initial_moisture_concentration)

def rhs(time_s: float, state: NDArray[np.float64]) -> NDArray[np.float64]:

temperature = state[:node_count]

concentration = np.exp(state[node_count:])

radius = float(interpolate_radius(time_s, radius_history))

density = density_q4(concentration)

heat_capacity = heat_capacity_q4(concentration)

conductivity = thermal_conductivity_q4(concentration)

diffusivity = diffusivity_q4(concentration, temperature)

ambient_temperature = interpolate_history(time_s, history.time_s, history.temperature_c)

ambient_moisture = interpolate_history(time_s, history.time_s, history.moisture_concentration)

temperature_div = radial_flux_divergence(

temperature, conductivity, 1.0, radius * parameters.heat_transfer_coefficient_w_m2_k,

ambient_temperature,

) / radius**2

moisture_div = radial_flux_divergence(

concentration, diffusivity, 1.0, radius * parameters.mass_transfer_coefficient_m_s,

ambient_moisture,

) / radius**2

return np.concatenate([temperature_div / (density * heat_capacity), moisture_div / concentration])

initial_state = np.concatenate([temperature_initial, np.log(moisture_initial)])

atol = np.concatenate([np.full(node_count, 1.0e-8), np.full(node_count, 1.0e-10)])

event = None

if maximum_moisture_threshold is not None:

def event(time_s: float, state: NDArray[np.float64]) -> float:

del time_s

return float(np.max(np.exp(state[node_count:])) - maximum_moisture_threshold)

event.terminal = True

event.direction = -1.0

result = solve_ivp(

rhs, (start_time, float(output_times[-1])), initial_state, method="BDF", t_eval=output_times,

rtol=relative_tolerance, atol=atol, max_step=max_step_s, jac_sparsity=sparsity, events=event,

)

if not result.success:

raise RuntimeError(f"Problem 4 solver failed: {result.message}")

result_time = result.t

result_state = result.y

if maximum_moisture_threshold is not None:

if not result.t_events or result.t_events[0].size == 0:

raise RuntimeError("Maximum moisture threshold was not reached")

event_time = float(result.t_events[0][0])

event_state = result.y_events[0][0]

if result_time.size and np.isclose(result_time[-1], event_time, atol=1.0e-9, rtol=0.0):

result_time = result_time.copy(); result_state = result_state.copy()

result_time[-1] = event_time; result_state[:, -1] = event_state

else:

result_time = np.append(result_time, event_time)

result_state = np.column_stack([result_state, event_state])

return Problem4Solution(

time_s=np.asarray(result_time), xi=xi,

temperature_c=result_state[:node_count].T,

moisture_concentration=np.exp(result_state[node_count:].T),

)

def sample_moving_solution(

solution: Problem4Solution,

radius_history: RadiusHistory,

radius_cm: ArrayLike,

) -> NDArray[np.float64]:

"""Sample moisture at fixed physical radii; outside cells are NaN."""

requested = np.asarray(radius_cm, dtype=float)

if requested.ndim != 1 or requested.size == 0 or np.any(~np.isfinite(requested)) or np.any(requested < 0.0) or np.any(np.diff(requested) <= 0.0):

raise ValueError("Requested radii must be strictly increasing")

radii_m = requested * 0.01

sampled = np.full((solution.time_s.size, requested.size), np.nan, dtype=float)

current_radius = interpolate_radius(solution.time_s, radius_history)

for row, radius in enumerate(current_radius):

valid = radii_m <= radius + 1.0e-12

if np.any(valid):

coordinates = radii_m[valid] / radius

sampled[row, valid] = np.interp(coordinates, solution.xi, solution.moisture_concentration[row])

return sampled

def sample_surface(solution: Problem4Solution) -> NDArray[np.float64]:

"""Return the actual moving-surface concentration."""

return solution.moisture_concentration[:, -1].copy()

def validate_radially_nonincreasing_moisture(

solution: Problem4Solution,

*,

tolerance: float = 1.0e-10,

) -> float:

"""Require the stored moving-coordinate profiles to decrease outward."""

if not np.isfinite(tolerance) or tolerance < 0.0:

raise ValueError("Tolerance must be finite and non-negative")

maximum_outward_increase = float(np.max(np.diff(solution.moisture_concentration, axis=1)))

if maximum_outward_increase > tolerance:

raise ValueError("Moisture profile contains an outward increase")

return maximum_outward_increase

def moisture_balance_diagnostics(

solution: Problem4Solution,

chamber_history: ChamberHistory,

radius_history: RadiusHistory,

*,

parameters: Problem1Parameters = Problem1Parameters(),

) -> dict[str, float]:

"""Check normalized-domain moisture change against the moving surface flux."""

if not np.isclose(solution.time_s[0], chamber_history.time_s[0], atol=1.0e-12, rtol=0.0):

raise ValueError("Moisture balance requires the initial-time row")

volumes = nodal_control_volumes(solution.xi.size, 1.0)

mean_moisture = solution.moisture_concentration @ volumes / np.sum(volumes)

ambient = np.interp(

solution.time_s,

chamber_history.time_s,

chamber_history.moisture_concentration,

)

current_radius = interpolate_radius(solution.time_s, radius_history)

mean_rate_from_surface = (

-2.0

* parameters.mass_transfer_coefficient_m_s

/ current_radius

* (solution.moisture_concentration[:, -1] - ambient)

)

integrated_change = cumulative_simpson(

mean_rate_from_surface,

x=solution.time_s,

initial=0.0,

)

residual = (mean_moisture - mean_moisture[0]) - integrated_change

total_change = abs(float(mean_moisture[-1] - mean_moisture[0]))

maximum_residual = float(np.max(np.abs(residual)))

return {

"maximum_absolute_residual": maximum_residual,

"final_residual": float(residual[-1]),

"relative_max_residual": maximum_residual / max(total_change, 1.0e-15),

"total_mean_moisture_change": float(mean_moisture[-1] - mean_moisture[0]),

"integrated_surface_change": float(integrated_change[-1]),

}

def density_shrinkage_compatibility(

solution: Problem4Solution,

radius_history: RadiusHistory,

) -> dict[str, object]:

"""Audit the hypothetical interpretation of Appendix 4 rho as wet density.

This is separate from the moisture PDE flux balance. With fixed length,

dry mass per unit length would be 2*pi*R**2*integral[rho(C)/(1+C)*xi dxi].

The first field must be the initial field; no conservation is imposed here.

"""

if not np.isclose(solution.time_s[0], radius_history.time_s[0], atol=1.0e-12, rtol=0.0):

raise ValueError("Density compatibility requires the initial-time row")

volumes = nodal_control_volumes(solution.xi.size, 1.0)

concentration = solution.moisture_concentration

implied_dry_density = density_q4(concentration) / (1.0 + concentration)

radii = interpolate_radius(solution.time_s, radius_history)

# Unit-radius annular volumes already include pi (their sum is pi).

mass_per_length = radii**2 * (implied_dry_density @ volumes)

ratio = mass_per_length / mass_per_length[0]

return {

"interpretation": "hypothetical wet bulk density; constant cylinder length",

"time_s": solution.time_s.tolist(),

"implied_dry_mass_per_length_kg_m": mass_per_length.tolist(),

"mass_ratio_to_initial": ratio.tolist(),

"final_mass_ratio": float(ratio[-1]),

"maximum_absolute_ratio_deviation": float(np.max(np.abs(ratio - 1.0))),

}